Equipment and materials for high-temperature ceramic welding

Units for high-temperature ceramic welding, output is up to 120 kg/hour. Ceramic powders for repair of dinas, chamotte, chromomagnesite and other linings of coking, glass-boiling, metallurgical and other furnaces. The repair is made without stopping and cooling furnaces. CERAMIC WELDING ENABLES REPAIRS OF HIGH TEMPERATURES LININGS FOR OVENS WITHOUT ANY STOPS. ### SAFETY, LONG SERVICE LIFE, EFFICIENCY OF YOUR OVENS #### POWDERS FOR REPAIR WORKS. MARK; PURPOSE. “Soyuz”; For repair and maintenance of coke ovens and batteries and glassmaking furnaces. Composition and thermal properties of welding on mass are like silica refractory.. “C”; Repair and maintenance of magnesite (general) lining and ladles. For lining and repair of steel casting ladles.. “T”; For lining and repair of steel casting ladles and glassmaking furnaces.. “2S”; For lining, repair and design modifications of ovens. A very high adhesion degree. Two layer variant of repair.. “MK”; For repair and reconstruction of glass making ovens, steel smelting ladles which lining consist of 70% AL2O3.; #### REPAIR EQUIPMENT. Performances; Model. UKN-50; UKN-120. Lance length, m; 2…4; 2…9. Range of capacity regulation, kg/hour; 20…50; 30…120. Max. powder change, kg; 30; 50. Distance of mixture feeding, m; to 20; to 50. Dimension BxLxH, mm; 570x1100x1300; 570x1100x1300. Mass, kg; 100; 130. Team of repair workers; 3; 3. ### we offer: - Equipment delivery - Powder repair delivery - Repair and inspection of lining - Repair of equipment - Personal training.